GAHM2024 Use Cases

Rick Luettich 1/25/2026

This document provides an overview of use cases for the GAHM2024 parametric tropical cyclone model and it associated capabilities including the application of land roughness and blending with a gridded meteorological data set.  In each case the required Matlab scripts, inputs and outputs are described.

The following use cases are considered:

- Compute GAHM2024 parameter values and write to an expanded Best Track output file

- Compute GAHM2024 wind velocity and pressure values at a series of longitude, latitude positions, with or without blending and land roughness.  This case computes the values at exactly the specified input points.

- Compute GAHM2024 wind velocity and pressure values on a regular grid of longitude, latitude points.  This case computes GAHM2024 values on a radial grid and interpolates the results to the regular grid.

Use Case 1: Compute and output GAHM2024 parameter values

Matlab script: run_build_GAHM2024_param_file.m

%-------------------------------------------------------------------------------------

% script to run build_GAHM2024_param_file.m for a specified number of

% storms and either GAHM version 3 or version 4

%

% write output to fort.22 like file. WARNING: While the output file has a

%     similar format to ADCIRC'S fort.22, it should not be used with GAHM

%     (NWS=20) in ADCIRC !!!! Rmax and Bg values are not strictly

%    compatible with the GAHM implementation in ADCIRC.

%

%

%                    7/9/2025  Rick Luettich

%--------------------------------------------------------------------------

The following control parameters or inputs can be set in the script and are required to run this script:

%% GAHM constants

GAHM_constants.Vmax_multiplier=1;  % =1 use full Vmax, =0.9 use 90% Vmax, etc.

GAHM_constants.one2tenF=0.89; %convert 1-min windspeed to 10-min wind speed

GAHM_constants.BLF=0.9; %wind speed reduction from top of boundary layer to 10m

GAHM_constants.Bmin=0.5; %minimum allowable value of original Holland B

GAHM_constants.Bmax=2.5; %maximum allowable value of original Holland B

GAHM_constants.SVorMax_10_tblmin=20; %minimum allowable gradient wind 									velocity at the top of the boundary layer

GAHM_constants.SVorQuad_10_tblmin=5; %minimum allowable quadrant wind 								velocity at the top of the boundary layer

GAHM_constants.rhoa=1.204; %density of air (kg/m3)

GAHM_constants.pback_def=1013; %default background pressure if not specified in 							environmental field

GAHM_constants.version=4; %GAHM2024 version to use, v3 – Rick’s iterative solver, 						v4 – Matlab solvers

GAHM_constants.Bg0M=1.05; %=Bg/M used for iterative solver

GAHM_constants.c0=0; % c0 used for iterative solver

%% track file info

trackfile_name = "ibtracs.NA.list.v04r01.csv"; % track file name containing best track 					type input data for GAHM2024 (e.g., position, Vmax, central 					pressure, Vquadrants, etc)

trackfile_type = "IBTrACS"; % specific format of track file input.  Choices are “IBTrACS”, 						“ATCF”, ???

%% storms to process – more than one can be specified

storm(1).name = 'FLORENCE';

storm(1).year = '2018';

storm(1).designation = 'AL06';

storm(1).starttime=0 or '2018091312'; % if 0, start at the first time in the track file, 					otherwise, start at the specified time.  This must match a time 					that is present in both the track file and the Environmental 					input file (if used).

storm(1).endtime=0 or '2018091412'; % if 0, end at the last time in the track file, 						otherwise, end at the specified time.

%% type of environmental field

env.type = 1; % =1 use translation velocity as in ASWIP / ADCIRC; = 2 use translation 				velocity folloiwng Lin and Chavez (ref); = 3 read in from gridded file in 				.mat format

env_file_name='ERA5_output.mat'; % .mat file contining gridded environmental 	fields, 				needed if env.type=3;

Use Case 2: Compute GAHM2024 wind velocity and pressure values at a series of longitude, latitude positions.  This case computes the values at exactly the specified input points.  Options include:

- three choices for the Environmental field

- GAHM only output or include land roughness

- GAHM (and roughness) blended with large scale wind and pressure fields.