GAHM2024 derivation*

Rick Luettich 8/1/2025

*adapted from J. Gao (2018)

Consider the radial profile of atmospheric pressure in a tropical cyclone can be expressed as a hyperbolic function of the deviation of the pressure from an ambient, far-field pressure         (), Holland (1980).

Pr=Pc+Pn-Pce-ArBg#1

P(r) is the atmospheric pressure (mb) at radius r, Pc is the minimum central pressure (mb) in the eye of the cyclone, Pn is the ambient, far-field pressure, and e is the exponential function.  A and Bg are scaling parameters.

Substituting this expression into the gradient wind balance in radial coordinates yields an equation for the tangential gradient wind

Vgr=ABgPn-Pce-ArBgρairrBg+rf22-rf2#2

where  Vg(r) is the gradient wind (m/s) at radius r (m), ρair is the density of air (kg/m3), and f is the Coriolis parameter (s-1).

Setting Vgr=Rmw=Vmax, where Rmw (m) is the radius to maximum winds and Vmax (m/s) is the maximum wind speed, and rearranging terms, Eq (2) can be rewritten as:

ABgPn-Pcρair=eARmwBgRmwBgVmax+ Rmwf22- Rmwf22#3

Letting A≡φ RmwBg and Ro≡VmaxRmwf  and rearranging yields

ABgPn-Pcρair=eφRmwBgVmax21+Ro-1#4

substituting Eq (4) into Eq (2) yields:

Vgr=Vmax21+Ro-1RmwrBgeφ1-RmwrBg+rf22-rf2#5

A further constraint can be introduced by requiring that dVgrdr=0  at r=Rmw.  To  compute the derivative of Eq (5), first rearrange the equation as:

Vgr +rf22=Vmax21+Ro-1RmwrBgeφ1-RmwrBg+rf22

and take derivatives of both sides (with the help of https://www.derivative-calculator.net):

2Vgr +rf2dVgrdr+f2=Vmax21+Ro-1BgφRmwrBg-1RmwrBgeφ1-RmwrBgr+rf22#

Setting  dVgrdr=0, Vgr=Vmax @ r=Rmw yields:

Vmaxf +Rmwf22=Vmax21+Ro-1Bgφ-1Rmw+Rmwf22#6

which can be simplified to:

Rmwf=Vmax1+Ro-1Bgφ-1          or          11+Ro=Bgφ-1

and therefore:

Bg=11+Roφ-1       or       φ=1+1Bg1+Ro#7

Eliminating φ, Eq. (4) can be re-written as:

Bg=e1+1Bg1+RoVmax21+Ro-1Pn-Pcρair-11+Ro#8

and Eq. (5) can be re-written as:

Vgr=Vmax21+Ro-1RmwrBge1+1Bg1+Ro1-RmwrBg+rf22-rf2#9

Rewriting Ro

Ro≡VmaxRmwf=VmaxrfrRmw= Rr rRmw= Rr c-1   where   Rr≡Vmaxrf   and    c=Rmwr#10

allows Eqs (8) and (9) to be written as:

Bg=e1+1Bg1+Rrc-1Vmax21+c Rr-1Pn-Pcρ-11+Rrc-1#11

Vgr=Vmax21+cRr-1cBge1+1Bg1+Rrc-11-cBg+rf22-rf2#12

For given values of Pn-Pcρ, Vmax, and Vgr, Eqs. (6) -(8) or alternatively Eqs (9) and (10) can be solved recursively to determine values of Bg and Rmw, respectively.  The latter equations involve only the unknowns Bg and c, including the constraint that 0<c≤1.

Finally, Eqs (6) and (9) can be written in terms of the original Holland (1980) B parameter,

B≡ Vmax2ePn-Pcρair

yielding:

Bg=Be1Bg1+Ro1+Ro-1-11+Ro #(13)

and

Bg=Be1Bg1+Rrc-11+c Rr-1-11+Rrc-1 #14

making it clear that  Bg→B   as  Ro=Rr c-1 → ∞

Dividing Eq (13) by B the resulting equation can be plotted as :

Profiles of BgB from GAHM with respect to log10Ro for different B values. (Figure 2.1 from Gao 2018).

Also, Eq (9) or (12) can be written in a non-dimensional form:

VgrVmax=1+Ro-1RmwrBge1+1Bg1+Ro1-RmwrBg+rRmw212Ro2-rRmw12Ro#15

VgrVmax=1+cRr-1cBge1+1Bg1+Rrc-11-cBg+12Rr2-12Rr#16

and plotted as shown below.

Normalized gradient wind profiles for the original Holland (1980) model and GAHM for log10Ro=0, 1, and 2 (or correspondingly  Ro =1, 10, and 100) and B  Bg? varying from 0.5 to 2.  (Figure 2.3 from Gao 2018).

As a check, setting r=Rmw in Eq (15) yields

VgrVmax=1+Ro-1+12Ro2-12Ro

VgrVmax=12Ro4RoRo+1+1-12Ro

VgrVmax=12Ro2Ro+12-12Ro=1

It is assumed that Vgr,  Vmax are tangential, 10 min average vortex velocities at the top of the boundary layer.  These are equal to the 10-min average vortex speed since it is assumed that the vortex has only tangential velocity at the top of the boundary layer.

It is helpful to use GAHM to compute the pressure deficit, which is easily derived from Eqs (1).

Pr-Pn =Pn-Pce-ArBg-1=Pn-Pce-φRmwrBg-1#1

Alternative derivation of dVgrdr

With the help of https://www.derivative-calculator.net

dVgrdr=Vmax21+Ro-1BgφRmwrBg-1RmwrBgeφ1-RmwrBgr+rf222Vmax21+Ro-1RmwrBgeφ1-RmwrBg+rf22-f2#

Which using Eq (5) is equal to

dVgrdr=Vmax21+Ro-1BgφRmwrBg-1RmwrBgeφ1-RmwrBgr+rf222Vgr +rf2-f2#

Setting  dVgrdr=0, Vgr=Vmax @ r=Rmw yields:

fVmax +Rmwf22=Vmax21+Ro-1Bgφ-1Rmw+Rmwf22#

which is identical to Eq (6).

Deriving Vgr and Vmax from forecast / best track / ibtracs data files.

Data sources for GAHM2024 are typically comprised of values for the eye position (lon, lat), the minimum central pressure (mb), Pc, the ambient pressure (mb), Pn, (in ADCIRC this is assumed to be 1013 mb), the 1-min sustained maximum total windspeed at 10m height above ground (kt), Vmax⁡_1_10, and the radial distances (nautical miles) from the TC eye to the 1-min sustained, total 64, 50, and 34 knot isotachs in the NE, SE, SW, and NW quadrants.  Depending on the data source, the radial distance to the radius to maximum winds (nautical miles), Rmw, may also be provided.

To utilize this information in GAHM2024, the total wind velocities must be converted into 10-min average, tangential velocities at the top of the boundary layer.  This is done utilizing the following assumptions and notation.

- Define values that satisfy the GAHM2024 equations: It is assumed that the GAHM2024 equations apply to 10-minuite averaged “vortex fields” at the top of the tropical cyclone boundary layer (and thus above the influence of friction with the land/water surface).  These are designated Pc_vor_10_tbl,Vvor_10_tbl  where the _vor subscript denotes “vortex field”, the initial _10 subscript denotes 10-min average, and the latter _tbl subscript denotes a height equal to the top of the boundary layer.

- Convert 1-min sustained wind velocities to 10-min averaged values: It is assumed that the maximum, Vmax, and the 64, 50 and 34 knot isotach input wind speeds are 1-min sustained values and that these can be converted to 10-min averaged values using a conversion factor one2ten = 0.89 (ref), e.g., Vmax⁡_10= one2ten*Vmax⁡_1, where the initial _1 subscript denotes a 1-minute sustained value.  It is assumed that the input pressure values are 10-min averages and need no conversion.

- Convert wind velocities from 10 m above ground to the top of the boundary layer: It is assumed that all wind velocities at the top of the boundary layer and 10 m above ground are related via a boundary layer factor, BLF = 0.9 (or other), i.e., V⁡_10_10= BLF*V⁡_10_tbl, and that in the northern hemisphere the vortex component of the velocity experiences a counter clockwise turning angle (clockwise in the southern hemisphere) defined as:

Turning angle (deg) =10*r/Rmw  if r≤Rmw

Turning angle (deg) =10+75*(r/Rmw-1)  if Rmw<r≤1.2*Rmw=

Turning angle (deg) =25  if 1.2*Rmw<r

- Decompose the total pressure field and total wind velocity field into “vortex fields” and “environmental fields”: It is assumed that the input wind velocity and pressure fields are total fields that represent the sums of “vortex fields”, Pc_vor_10_10,Vvor_1_10, and “environmental fields”, Pc_env_10_10,Venv_10_10, reflecting larger scale spatial processes.  Due to the linear scaling between 10 m elevation and the top of the boundary layer, this separation is applicable either at a height of 10 m above ground, i.e., Vmax⁡_10_10= Vmaxvor⁡_10_10+ Venv⁡_10_10, or at the top of the boundary layer, i.e., ., Vmax⁡_tbl= Vvor⁡_10_tbl+ Venv⁡_10_tbl.

- The “environmental fields”, Pc_env_10,Venv_10, may either be specified as spatially varying inputs, e.g., if extracted from large scale gridded model data, or inferred from the storm’s translation velocity, Vtrans.  In the latter case, two options are supported:

  - ADCIRC assumes Venv_1_10 =1.5* Vtrans0.63  (for Vtrans in knots) or Venv_1_10 =1.5* Vtrans0.63*0.514440.37  (for Vtrans in m/s) and Venv_1_10  varies with radial distance from the eye following the same relationship as Vgr, Eq. (15) or (16).  Below we assume that an environmental velocity derived from the translation velocity better reflects a 10-min average than a 1-min sustained value and therefore that Venv_10_10 =1.5* Vtrans0.63 or Venv_10_10 =1.5* Vtrans0.63*0.514440.37.

  - Lin and Chavez (ref) analyzed H*Wind snapshots and concluded that a spatially constant Venv_xx_10 =0.6*Vtrans where Venv_xx_10 is rotated 20 degrees counter clockwise from Vtrans, is better supported than the value assumed in ADCIRC.  I do not recall how these authors interpreted the time scale associated with the translation velocity, e.g., 1-min sustained or 10-min average.  Below we assume that an environmental velocity derived from the translation velocity better reflects a 10-min average than a 1-min sustained value and therefore that Venv_10_10 =0.6*Vtrans.where Venv_10_10 is rotated 20 degrees counter clockwise from Vtrans.

To proceed,

- Compute the 10 min averaged, maximum, tangential vortex velocity at 10 m elevation, VMaxvor⁡_10_10, from the input total maximum velocity magnitude, Vmax_1_10  and the environmental velocity, Venv_10_10 .  The total maximum velocity occurs when the environmental velocity and the maximum, tangential vortex velocity are aligned, i.e.,

Vmax_10_10=one2ten*Vmax_1_10 *Venvuv_10_10

where Venvuv⁡_10_10 is the unit vector in the direction of Venv⁡_10_10.  Because of this alignment, the magnitude of the maximum, tangential vortex velocity is equal to the difference between the magnitudes of the input total maximum velocity and the environmental velocity.

Vmaxvor_10_10=Vmax_10_10- Venv_10_10

The maximum vortex velocity at the top of the boundary layer is thus equal to

Vmaxvor⁡_10_tbl= Vmaxvor⁡_10_10BLF*Vvoruv_tbl

where the vortex velocity unit vector at the top of the boundary layer, Vvoruv_tbl, is tangent to the circular vortex and directed ccw in the northern hemisphere (cw in the southern hemisphere) at any location.  The location around the vortex where Vmax_10_10 and Vmax_10_tbl occur will be different due to the assumed turning angle between 10 m and the top of the boundary layer.

- Check the magnitude of Vmaxvor⁡_10_tbl.  It is assumed that the gradient wind balance is valid only if Vmaxvor⁡_10_tbl≥Vmaxvor⁡_10_tblmin .  If this condition is violated, GAHM should not be used.  A reasonable value for Vmaxvor⁡_10_tblmin=20 kts????

- Assuming the availability of isotach total wind magnitudes (i.e., VQuad_1_10= 64, 50, 34 kts at specified radial distances from the eye in the NE, SE, SW and NW quadrants), these need to be converted to 10-min, tangential vortex velocities at the top of the boundary layer, VQuadvor_10_tbl, for use in GAHM2024.  We can write this as:

VQuadvor_10_tbl= VQuadvor_10_tbl* VQuadvoruv_tbl

where the unit vector in each quadrant, VQuadvoruv⁡_tbl, is assumed to be tangent to a circle at the midpoint of that quadrant and directed ccw in the northern hemisphere (cw in the southern hemisphere).  For example, in the northern hemisphere the unit vector for the NE quadrant would point NW (315 deg cw from North).  Given this, the quadrant vortex velocity at 10 m height is:

VQuadvor_10_10= BLF*VQuadvor_10_tbl*VQuadvoruv_10

where the quadrant unit vector at 10 m, VQuadvoruv_10, is equal to the value at the top of the boundary layer plus any assumed turning angle (see above).

Taking the isotach value as the input magnitude of the total wind velocity, this can be expressed as the magnitude of the sum of vortex and environmental components,

one2ten*VQuad_1_10=VQuad_10_10= VQuadvor_10_10+ Venv_10_10

which can be solved for the unknown VQuadvor_10_10.  Two consistency checks can be performed.  The first assumes that fitting the gradient wind balance to the isotach is valid only if VQuadvor⁡_10_tbl≥VQuadvor⁡_10_tblmin.  Setting VQuadvor⁡_10_tbl=VQuadvor⁡_10_tblmin  , the above equation can be evaluated to determine a minimum acceptable value of VQuad1_10min.  If VQuad1_10<VQuad1_10min, the isotach should not be used to determine Rmw.  The second assumes that VQuadvor⁡_10_tbl≤Vmaxvor⁡_10_tbl.  Setting VQuadvor⁡_10_tbl=Vmaxvor⁡_10_tbl , the above equation can be evaluated to determine a maximum acceptable value of VQuad1_10max.

Squaring both sides and expanding the right-hand side yields a quadratic equation for VQuadvor_10_10:

VQuadvor_10_102+2VQuadvor_10_10Venv_10_10VQuadvoruv_10_10 ∙Venvuv_10_10

+Venv_10_102-VQuad_10_102=0

which can be solved using the quadratic formula with the coefficients:

a=1

b=2Venv_10_10VQuadvoruv_10_10 ∙Venvuv_10_10

c=  Venv_10_102-VQuad_10_102

This places a lower bound on VQuad_10_10 (or equivalently an upper bound on Venv_10_10) to ensure that VQuadvor_10_10>0.

There are several cases that can occur:

- If c<0, the term under sqrt > 0 and the sqrt will be greater than b.  In this case the sum of the (+) sqrt and (-b) is guaranteed to be greater than 0, regardless of the sign of b.

- If c=0, then sqrt = |b|.  The (+) sum of this term and b will be greater than or equal to zero, regardless of the sign of b.

- If c > 0, then the sqrt will in all cases be less than b and in some locations around the vortex it will be imaginary (e.g., in locations where the environmental and vortex velocities approach orthogonality, causing the dot product of their unit vectors and b to approach zero.)  This will lead to unphysical negative (if b > 0) or imaginary (regardless of the sign of b) magnitudes for VQuad_10_10.

The details for Case iii depend on the spatial structure of Venv_10_10.

Iii a. if Venv_10_10 has a uniform direction, as assumed in ADCIRC/ASWIP or Lin & Chavez, then VQuad_10_10≥ Venv_10_10 to ensure a viable solution around the full vortex.  Since GAHM is only fit along the four radial lines that bisect the four quadrants, the negative case is more likely to occur than the imaginary case.  If Venv_10_10 varies in space, as assumed in ADCIRC/ASWIP, and the distance to the specified isotach varies from quadrant to quadrant, the relationship between the environmental and quadrant velocity magnitudes may also vary from quadrant to quadrant.

iiib. In the more general case where the direction and magnitude of Venv_10_10 vary in space, it appears there is no choice than to evaluate the relationship and adjust accordingly in each quadrant.

If the environmental velocity varies slowly across the vortex, Case iii is most likely to occur for the 34 kt isotach and therefore the consequences of any adjustment imposed on the solution will extend beyond that isotach.

It is not possible to fit the GAHM equations to a case where VQuadvor_10_10=0; therefore any adjustment made in response to Case iii, must result in result in VQuadvor_10_10> 0.  Adjusting either the environmental or quadrant velocities ends up determining the value of VQuadvor_10_10.  If the environmental velocity is adjusted, this must be carried throughout the calculations, including when GAHM is evaluated.  If the quadrant velocity is adjusted (which is functionally equivalent to adjusting VQuadvor_10_10 if the environmental velocity is not modified), then GAHM will not recover the quadrant velocity.  In any case, the resulting value of VQuadvor_10_10 will directly affectRmw and Bg obtained from GAHM2024.  It appears that a small value of VQuadvor_10_10 results in an unreasonably small (outlier) value for Rmw.

A special variant arises in the ADCIRC / ASWIP implementation of GAHM, because the radial profile of Venv_10_10 is assumed to match the radial profile of the vortex gradient wind.  Thus at the radial location where VQuad_10_10 has been specified, the environmental velocity can be expressed as:

Venv_10_10= V*env_10_10*VQuadvor_10_10VMaxvor⁡_10_10

where V*env_10_10 is the environmental velocity at r=Rmw.  In this case the quadratic equation can be written as:

VQuadvor_10_1021+2VQuadvoruv_10_10 ∙Venvuv_10_10V*env_10_10VMaxvor⁡_10_10+                                                                             V*env_10_102VMaxvor⁡_10_102  -VQuad10102=0

VQuadvor_10_102VQuadvoruv_10_10+Venvuv_10_10V*env_10_10VMaxvor_10_102   -VQuad_10_102=0

VQuadvor_10_10 =VQuad_10_10VQuadvoruv_10_10+Venvuv_10_10V*env_10_10VMaxvor_10_10

In contrast to the general case, this variant imposes no explicit constraint on the relative size of VQuad_10_10 and V*env_10_10.

The previous checks ensured that VQuadvor_10_10>0.  It is also necessary to check that VQuadvor_10_10≤VMaxvor_10_10.  If this is not the case a further adjustment must be made.  Setting VMaxvor_10_10=VQuadvor_10_10 while maintaining the environmental velocity, raises the total maximum velocity in the storm and ensures that Rmw= distance to the specified isotach in the specified quadrant.  Alternatively, since 
VMaxvor_10_10=Vmax_10_10- Venv_10_10, the magnitude of the environmental velocity could be reduced until VQuadvor_10_10=VMaxvor_10_10.  This requires the adjusted environmental velocity to be carried throughout all of the GAHM calculations and again ensures that Rmw= distance to the specified isotach in the specified quadrant.

Input required for GAHM2024

Vmax – maximum wind speed, assumed to represent the vector sum of the maximum vortex wind velocity and the environmental wind velocity.  In IbTracs file, ATCF file, fort.22 file, this is assumed to be in knts.  GAHM2024 requires it to be converted to m/s.

RQuad – radial distances to 34, 50, 64 kt isotach in the 4 quadrants.  In IbTracs file, ATCF file, fort.22 file, this is assumed to be in nautical miles.  GAHM2024 requires it to be converted to meters.

Environmental velocity @ location of Vmax and at location of RQuad values.  GAHM2024 requires this to be in m/s.  Options for determining the environmental velocity are generating it from the translation velocity or reading it in from a gridded source

Environmental velocity type – specifying to use ADCIRC version, Lin and Chavez (2012) version, or read in from file version.  envtype=1, 2, 3, respectively.

Rmax – default value for Rmax (in meters).  This is only used if there are no RQuad values.

Pn-Pc – central pressure deficit in Newtons / m^2 = 100*mb

ρair - density of air (kg/m3), default 1.2

Bmin,  Bmax- minimum and maximum allowable values of the Holland (1980) B parameter, default 0.25, 2.5

BLF – scaling factor reducing the wind velocity at the top of the boundary layer to the value at 10 meters, range 0.75 – 0.9, default 0.9?

one2tenF – scaling factor reducing the wind speed from a 1-min sustained value to a 10-min average value, default 0.89.

Pn = Pback

Blending GAHM2024 with gridded data

It is assumed that GAHM2024 wind and pressure fields will be blended with gridded data.  To do so the following is required:

An environmental field that is subtracted from Vmax and the quadrant isotach velocities.  This should have as little vortex in it as possible?  An estimate of Pc-Pn from the original gridded data and Pc-Pn from the extracted hurricane field.  The ratio can be used to scale Pc-Pn from the IBtracs file to compensate for any vortex component in the environmental field.

GAHM2024 default conditions

If no isotach distances exist in a quadrant, Rmw for that quadrant is determined as the average value from the quadrant(s) having valid values.  In this case Bg is determined from Eq. (13).

isotach indices are 1=34kt, 2=50kt, 3=64kt, 4=00kt

for itime =1:ntimes

(itime).flag00(q=1:4) = 0 if have at least one good isotach in one quadrant

(q=1:4)= 2 if have no good isotachs in any quadrant

(itime).Rmax00(q=1:4) = 0 if have at least one good isotach in one quadrant

(q=1:4) = Rmax default (from track file) if no good isotachs in any quadrant

(itime).Bg00(q=1:4) = 0 if have at least one good isotach in one quadrant

(q=1:4) =Bg computed from GAHM2024 if no good isotachs in any quadrant

(itime).VmaxVor_10_tbl00(q=1:4)= 0 if have at least one good isotach in one quadrant

(q=1:4) =computed if no good isotachs in any quadrant

(itime).flag34(q=1:4) = 0 if have no good isotachs (34,50,64) in any quadrant

(q) = 1 if good 34 kt isotach in quadrant q

(q) = 2 if no 34 kt isotach in quadrant q

(itime).Rmax34(q=1:4) = 0 if have no good isotachs (34,50,64) in any quadrant

(q) = Rmax computed from GAHM2024 if good 34 kt isotach in quadrant q.

(q) = Rmax default (from Rmax avg) if no good 34 kt isotach in quadrant q.

(itime).Bg34(q=1:4) = 0 if have no good isotachs (34,50,64) in any quadrant

(q) =Bg computed from GAHM2024 if any quadrant has a good 34 kt isotach

(itime).VmaxVor_10_tbl34(q=1:4)= 0 if have no good isotachs (34,50,64) in any quadrant

(q) =computed if good 34 kt isotach in quadrant q

(q) =computed if no good 34 kt isotach in quadrant q

condition 1: If VmaxVor_10_tbl is too small (e.g., 10 m/s?) – can’t use GAHM2024, (itime).flag00(q=1:4)=9 (all other flags also =9).

condition 2: If VQuad_Vor_10_10(iso,q)  < VQuad_Vor_10_10min (perhaps 5 m/s?) assume it is missing and needs to be replaced by a default value, (itime).flagISO(q)=3

condition 3: If VQuad_Vor_10_10(iso,q) > VmaxVor_10_10 – assume it is = VmaxVor_10_10 or is missing and needs to be replaced by a default value? (itime).flagISO(q)=4.

(itime).flag50(q=1:4) = 0 if have no good isotachs (34,50,64) in any quadrant

(q) = 1 if have a good 50 kt isotach in quadrant q

(q) = 2 if have no 50 kt isotach in quadrant q

(itime).flag64(q=1:4) = 0 if have no good isotachs (34,50,64) in any quadrant

(q) = 1 if have a good 64 kt isotach in quadrant q

(q) = 2 if have no 64 kt isotach in quadrant q

ATCF2_data(itime).numiso

ATCF2_data(itime).flag00(q), flag34(q), flag50(q), flag64(q)

ATCF2_data(itime).R00(q), R34(q), R50(q), R64(q)

ATCF2_data(itime).Rmax00(q), Rmax34(q), Rmax50(q), Rmax64(q)

ATCF2_data(itime).Bg00(q), Bg34(q), Bg50(q), Bg64(q)

ATCF_data_extended(itime).VmaxVor_10_tbl00(q), VmaxVor_10_tbl34(q), 							 VmaxVor_10_tbl50(q), VmaxVor_10_tbl64(q)

ATCF2_data(itime).RmaxQ(q)

References

One2ten=0.88, IBTracs - Kurk, M.C., K.R. Knapp, D.H. Levinson, 2010. A Technique for Combining Global Tropical Cyclone Best Track Data, J Atmos Ocean Tech, v27, pgs 680-692, DOI: 10.1175/2009JTECHA1267.1